Using Python (Netmiko) to manage network devices

A simple way to start network automation with python.

Prerequisites

- Visual Studio Code (We already installed this)

- Python 3.8+(We already installed this)

- Network devices (Router, Switch, …) (We need this)

- Python library Netmiko (We need this)

1. Obtain/Access A Network Device

Since we don’t have access to physical network devices, we will use routers and switches provided by Cisco and made available in the cloud for FREE .

Go to the Cisco DevNet Sandbox portal DevNet auth, register and launch:

“Catalyst 9000 Always-On Sandbox”

Copy the following:

- Hostname / IP :  devnetsandboxiosxec9k.cisco.com

- Username :  cyprien.temateu

- Password :  _02Flsg2NrA_B

These are the credentials your Python script will use later.

2. Set Up Your VS Code Workspace

You do not use Remote-SSH here.

Instead:

- Open VS Code

- Create a new file for your automation scripts e.g: cisco_connect.py

- Open a terminal inside VS Code and Install Netmiko: pip install netmiko

3. Write a Python Script to SSH into the Sandbox (Remote Cisco Network Device)

Netmiko is a python library used to ssh and manage network devices (Cisco, Nokia, …)

from netmiko import ConnectHandler

# replace with your sandbox details

device = {

"device_type": "cisco_xe",

"host": "your_sandbox_host",

"username": "your_username",

"password": "your_password",

}

connection = ConnectHandler(**device)

output = connection.send_command("show ip interface brief")

print(output)

connection.disconnect()

4. Test Your Script

From the terminal, run: python cisco_connect.py

You should see output like:

Interface              IP-Address      OK? Method Status Protocol

GigabitEthernet1       10.10.20.48     YES DHCP   up     up

...

5. Expand Into Automation

Explore how to use Netmiko:

- Sending multiple commands

- Configuring interfaces

- Backing up configs

- Parsing output